# Blynk.md

## Table of Contents

- Blynk.md

  - Table of Contents

  - Prerequisites

  - Creating a Blynk Device

  - Connecting Blynk to the TM4C

  - Updating ESP8266

## Prerequisites

- Hardware is setup (TM4C hooked up to ESP8266 via UART)

- ESP8266 is flashed with latest firmware

  - See section Updating ESP8266

## Creating a Blynk Device

- Sign into Blynk

- Create a template for your device

  - Click the Template icon in the left taskbar

  - Click the + New Template button in the top right corner of the screen

  - Name your template

  - Select ESP8266 as the hardware

  - Click Done

- Create datastreams for your widgets to connect to

  - Click the Datastreams tab at the top of the screen

  - Click the + New Datastream button in the top right corner of the screen

  - Name the datastream and select the virtual pin that it corresponds to

  - Edit the rest of the datastream’s settings as you like

  - Click Create

- Create a dashboard

  - Click the Datastreams tab at the top of the screen

  - Drag widgets from the widget box on the left of the screen

  - Click a widget’s gear icon to edit it

  - Name the widget and set its corresponding datastream

  - Click Save

- Create a device

  - Save the template you created by clicking the Save button in the top right corner of the screen

  - Click the Search icon in the left taskbar

  - Click the + New Device button in the top right corner of the screen

  - Select the From template option

  - Select the template you created

  - Name the device

  - Copy the BLYNK_AUTH_TOKEN from the top right corner of the screen

- Create a mobile dashboard

  - Log into the mobile app

  - Click the device that you created earlier

  - Click the wrench button in the top right corner of the screen to edit the dashboard

  - Click the + button in the top right corner of the screen to add a widget

  - Click the widget to edit it

  - Name the widget and set its corresponding datastream

  - Click the X button in the top left corner to exit the editor

## Connecting Blynk to the TM4C

- Create a Blynk device

- You are provided a BLYNK template ID, device name, and auth token

  - Only need the auth token

- Use the blynk_init function call to pass in your auth token, SSID, and password to connect to Blynk!

  - We suggest the SSID and PASS to correspond to the Lab IOT WiFi or a mobile hotspot

- Compile and flash program to TM4C

- If the HW is setup properly, then Blynk should say the device is online

## Updating ESP8266

- Download the Arduino IDE

  - Last tested version: 1.8.19

- Add ESP8266 indices to Arduino board manager URLs https://arduino.esp8266.com/stable/package_esp8266com_index.json

  - File > Preferences > Additional Board Manager URLs:

  - Go to Tools > Boards Manager

  - Search for esp8266

  - Install v3.0.2

- Add Blynk libraries

  - Tools > Manage Libraries...

  - Search for blynk, install Blynk by Volodymyr Shymanskyy v1.1.0

- Change board to esp8266

  - Tools > Board: "xxx" > ESP8266 Boards (3.0.2) > Generic ESP8266 Module

- Clone latest firmware

- Open up ESP_TM4C_Xfer_Rev_HandShake_NEW_BLYNK project

  - EE445L-F22-ESP8266-Blynk > ESP_TM4C_Xfer_Rev_HandShake_NEW_BLYNK > ESP_TM4C_Xfer_Rev_HandShake_NEW_BLYNK.ino

- Compile project (checkmark button)

- Select port to flash to

  - Tools > Port: "xxx"

  - Select open port, assuming only one USB device is currently connected

- Connect ESP8266 to ECE 445L ESP8266 flasher board (or similar UART-USB FTDI board)

- Upload using flash button (rightward facing arrow).